SOLAR PV SYSTEM PROPOSAL

WITH PPA FINANCING OPTIONS

Observatory Science Centre

3708 Main Road, Observatory, Cape Town

| Project Reference | SS-2026-SCI-002 |
| --- | --- |
| Date | 13 January 2026 |
| Valid Until | 12 February 2026 |
| System Size | 60 kWp (110 × 550W panels) |
| Configuration | Zero Export - Maximum Self-Consumption |

# 1. Executive Summary

This proposal outlines a 60 kWp grid-tied solar photovoltaic system designed for the Observatory Science Centre. The system has been specifically sized for ZERO EXPORT operation, ensuring maximum self-consumption and compliance with municipal grid connection requirements.

## Key Highlights

- Zero Export configuration - no grid feed-in required

- System sized to match peak daytime load (~50-55 kW)

- 92%+ self-consumption rate - minimal waste

- Two financing options: Cash Purchase or PPA

- NRS 097-2-1 compliant for City of Cape Town connection

# 2. Financing Options

## Option A: Cash Purchase

| Metric | Value |
| --- | --- |
| Total Investment (incl. VAT) | R 916 406,94 |
| Year 1 Savings | R 215 595,49 |
| Simple Payback Period | 4.3 years |
| 20-Year Net Benefit | R 8.4M |
| Asset Ownership | Immediate - you own the system |

## Option B: Power Purchase Agreement (PPA)

With a PPA, you pay ZERO upfront and only pay for the electricity generated at a rate lower than your current tariff.

| PPA Terms | Value |
| --- | --- |
| Upfront Cost | R 0.00 |
| PPA Term | 20 years |
| Starting PPA Rate | R 1.9574/kWh |
| Current Grid Rate | R 2.6099/kWh |
| Day 1 Discount vs Grid | 25% |
| PPA Annual Escalation | 6% |
| Expected Grid Escalation | 8% |
| Asset Ownership | Transfers to you after 20 years |

## Why PPA Rate Stays Below Grid Rate

The PPA escalates at 6% per year while Eskom/municipal tariffs historically increase at 8-12% per year. This means your savings grow over time:

| Year | Grid Rate | PPA Rate | Savings/kWh | Annual Savings |
| --- | --- | --- | --- | --- |
| Year 1 | R 2.6099 | R 1.9574 | R 0.6525 | R 53 899,00 |
| Year 5 | R 3.5507 | R 2.4712 | R 1.0795 | R 87 407,00 |
| Year 10 | R 5.2172 | R 3.3070 | R 1.9102 | R 150 833,00 |
| Year 15 | R 7.6658 | R 4.4255 | R 3.2403 | R 249 526,00 |
| Year 20 | R 11.2635 | R 5.9224 | R 5.3411 | R 401 135,00 |

# 3. 20-Year PPA Cash Flow Projection

| Yr | Generation | Self Used | Grid Rate | PPA Rate | Grid Cost | PPA Cost | Net Save |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 89 790 | 82 607 | 2.6099 | 1.9574 | 215 595 | 161 697 | 53 899 |
| 2 | 89 341 | 82 194 | 2.8187 | 2.0749 | 231 679 | 170 541 | 61 137 |
| 3 | 88 894 | 81 783 | 3.0442 | 2.1994 | 248 962 | 179 870 | 69 092 |
| 4 | 88 450 | 81 374 | 3.2877 | 2.3313 | 267 535 | 189 709 | 77 826 |
| 5 | 88 008 | 80 967 | 3.5507 | 2.4712 | 287 493 | 200 086 | 87 407 |
| 6 | 87 568 | 80 562 | 3.8348 | 2.6195 | 308 940 | 211 031 | 97 909 |
| 7 | 87 130 | 80 159 | 4.1416 | 2.7766 | 331 987 | 222 574 | 109 413 |
| 8 | 86 694 | 79 759 | 4.4729 | 2.9432 | 356 753 | 234 749 | 122 004 |
| 9 | 86 261 | 79 360 | 4.8307 | 3.1198 | 383 367 | 247 590 | 135 777 |
| 10 | 85 829 | 78 963 | 5.2172 | 3.3070 | 411 966 | 261 133 | 150 833 |
| 11 | 85 400 | 78 568 | 5.6346 | 3.5055 | 442 698 | 275 417 | 167 282 |
| 12 | 84 973 | 78 175 | 6.0853 | 3.7158 | 475 724 | 290 482 | 185 242 |
| 13 | 84 548 | 77 784 | 6.5722 | 3.9387 | 511 213 | 306 371 | 204 841 |
| 14 | 84 126 | 77 396 | 7.0979 | 4.1750 | 549 349 | 323 130 | 226 219 |
| 15 | 83 705 | 77 009 | 7.6658 | 4.4255 | 590 331 | 340 805 | 249 526 |
| 16 | 83 286 | 76 624 | 8.2790 | 4.6911 | 634 369 | 359 447 | 274 922 |
| 17 | 82 870 | 76 240 | 8.9414 | 4.9725 | 681 693 | 379 109 | 302 584 |
| 18 | 82 456 | 75 859 | 9.6567 | 5.2709 | 732 548 | 399 846 | 332 701 |
| 19 | 82 043 | 75 480 | 10.4292 | 5.5872 | 787 196 | 421 718 | 365 478 |
| 20 | 81 633 | 75 102 | 11.2635 | 5.9224 | 845 921 | 444 786 | 401 135 |
| TOTAL |  |  |  |  |  | R 5 620 090,62 | R 3 675 226,97 |

## PPA Summary

| 20-Year Total PPA Payments | R 5 620 090,62 |
| --- | --- |
| 20-Year Grid Cost (without solar) | R 9 295 317,59 |
| TOTAL SAVINGS OVER 20 YEARS | R 3 675 226,97 |
| Average Annual Savings | R 183 761,35 |
| NPV of Savings (10% discount rate) | R 1 138 143,76 |

# 4. System Sizing Justification

## 4.1 Why 60 kW (Not Larger)?

With zero export constraints, oversizing wastes money. The system is sized to match your peak daytime load:

| Parameter | Value |
| --- | --- |
| Monthly Consumption (from bill) | 8 703,48 kWh |
| Daily Average | 300.12 kWh/day |
| Estimated Peak Daytime Load | 45-55 kW |
| System Size Selected | 60 kW |
| Peak Solar Output (summer noon) | ~51 kW |
| Self-Consumption Rate | 92% |

## 4.2 Seasonal Performance

| Season | PSH | Daily Gen | Daily Load | Coverage | Notes |
| --- | --- | --- | --- | --- | --- |
| Summer (Dec) | 6.5 | ~320 kWh | ~380 kWh | 84% | Excellent |
| Autumn (Apr) | 4.5 | ~220 kWh | ~310 kWh | 71% | Good |
| Winter (Jun) | 3.3 | ~160 kWh | ~350 kWh | 46% | Grid needed |
| Spring (Sep) | 4.8 | ~240 kWh | ~300 kWh | 80% | Very Good |

# 5. Bill of Materials

## 5.1 PV Modules

| Description | Qty | Unit | Unit Price | Total |
| --- | --- | --- | --- | --- |
| JA Solar 550W Mono PERC Bifacial Module | 110 | ea | R 1 950,00 | R 214 500,00 |

## 5.2 Inverters

| Description | Qty | Unit | Unit Price | Total |
| --- | --- | --- | --- | --- |
| Solis S6-EH3P30K-H Three Phase Hybrid Inverter 30kW | 2 | ea | R 44 000,00 | R 88 000,00 |

## 5.3 DC Side Equipment (Panels to Inverters - 100m)

| Description | Qty | Unit | Unit Price | Total |
| --- | --- | --- | --- | --- |
| 6mm² Twin Core Solar DC Cable (6 string pairs × 100m) | 1200 | m | R 38,00 | R 45 600,00 |
| DC Surge Protection Device Type II (per MPPT) | 6 | ea | R 2 800,00 | R 16 800,00 |
| DC Isolator Switch 1000V 32A (per string pair) | 6 | ea | R 850,00 | R 5 100,00 |
| DC Combiner Box 3-string with Fuses | 2 | ea | R 4 500,00 | R 9 000,00 |
| MC4 Connector Pairs (per panel) | 110 | set | R 45,00 | R 4 950,00 |

## 5.4 AC Side Equipment (Inverters to Main CB - 20m)

| Description | Qty | Unit | Unit Price | Total |
| --- | --- | --- | --- | --- |
| 35mm² 4-Core Armoured Cable (per inverter run) | 80 | m | R 185,00 | R 14 800,00 |
| AC Surge Protection Device Type II | 2 | ea | R 3 200,00 | R 6 400,00 |
| AC Isolator 80A 4-Pole | 2 | ea | R 1 450,00 | R 2 900,00 |
| MCCB 63A 4-Pole 25kA | 2 | ea | R 3 200,00 | R 6 400,00 |
| Main Incomer MCCB 125A 4-Pole | 1 | ea | R 6 500,00 | R 6 500,00 |
| Solar AC Distribution Board (IP65) | 1 | ea | R 9 500,00 | R 9 500,00 |

## 5.5 Metering, Protection & Zero Export

| Description | Qty | Unit | Unit Price | Total |
| --- | --- | --- | --- | --- |
| Bi-directional Revenue Grade Energy Meter (CT Operated) | 1 | ea | R 18 500,00 | R 18 500,00 |
| Current Transformer Set 150/5A (3-phase) | 1 | set | R 3 800,00 | R 3 800,00 |
| Zero Export Device with CT (Grid Export Limiter) | 1 | ea | R 8 500,00 | R 8 500,00 |
| Earth Leakage Relay with CT | 2 | ea | R 3 800,00 | R 7 600,00 |
| Anti-Islanding Protection Relay (NRS 097 Compliant) | 1 | ea | R 8 500,00 | R 8 500,00 |

## 5.6 Communications & Monitoring

| Description | Qty | Unit | Unit Price | Total |
| --- | --- | --- | --- | --- |
| Solis Data Logging Stick WiFi/LAN | 1 | ea | R 6 500,00 | R 6 500,00 |
| Irradiance Sensor + Ambient Temp Probe | 1 | ea | R 8 500,00 | R 8 500,00 |
| Cloud Monitoring Platform Setup (1 year) | 1 | ea | R 3 500,00 | R 3 500,00 |

## 5.7 Mounting Structure

| Description | Qty | Unit | Unit Price | Total |
| --- | --- | --- | --- | --- |
| Aluminium Mounting Rails (per panel length) | 242 | m | R 185,00 | R 44 770,00 |
| Mid Clamps (2 per panel) | 220 | ea | R 28,00 | R 6 160,00 |
| End Clamps (4 per row) | 44 | ea | R 32,00 | R 1 408,00 |
| Adjustable Roof Mounting Brackets | 55 | ea | R 145,00 | R 7 975,00 |
| Complete Earth Bonding Kit (panels + rails) | 1 | set | R 4 500,00 | R 4 500,00 |

## 5.8 Sundries & Consumables

| Description | Qty | Unit | Unit Price | Total |
| --- | --- | --- | --- | --- |
| Hot-Dip Galvanised Cable Tray 150mm | 100 | m | R 185,00 | R 18 500,00 |
| Cable Glands & Termination Kit | 1 | set | R 2 200,00 | R 2 200,00 |
| Safety Labels & Signage (SANS compliant) | 1 | set | R 1 200,00 | R 1 200,00 |
| Installation Consumables (ties, fixings, etc.) | 1 | lot | R 6 500,00 | R 6 500,00 |

# 6. Cost Summary (Cash Purchase)

| Category | Amount (excl. VAT) |
| --- | --- |
| PV Modules | R 214 500,00 |
| Inverters (2 × 30kW) | R 88 000,00 |
| DC Equipment | R 81 450,00 |
| AC Equipment | R 46 500,00 |
| Metering, Protection & Zero Export | R 46 900,00 |
| Communications | R 18 500,00 |
| Mounting Structure | R 64 813,00 |
| Sundries | R 28 400,00 |
| Equipment Subtotal | R 589 063,00 |
| Margin (20%) | R 117 812,60 |
| Equipment Total | R 706 875,60 |
| Installation (60,000W × R1.50/W) | R 90 000,00 |
| PROJECT SUBTOTAL | R 796 875,60 |
| VAT (15%) | R 119 531,34 |
| GRAND TOTAL | R 916 406,94 |

# 7. Terms and Conditions

## 7.1 Cash Purchase Terms

- 50% deposit on acceptance

- 40% on delivery of equipment

- 10% on commissioning

## 7.2 PPA Terms

- No upfront payment required

- Monthly billing based on metered generation

- 20-year agreement with buyout options at years 7, 10, and 15

- Performance guarantee included

- Full maintenance and monitoring included

## 7.3 Warranty

- PV Panels: 12 years product, 25 years performance

- Inverters: 5 years manufacturer warranty (extendable)

- Workmanship: 2 years

## 7.4 Exclusions

- Grid connection fees (if applicable)

- Municipality application and approval fees

- Structural modifications to building

- Scaffolding or crane hire (if required)

- Battery storage system (available as add-on)

## 7.5 Validity & Lead Time

This quotation is valid for 30 days from the date of issue.

Estimated project duration: 3-4 weeks from deposit/agreement, subject to equipment availability.

ACCEPTANCE

Please indicate your preferred option:

| ☐ Option A: Cash Purchase | R 916 406,94 |
| --- | --- |
| ☐ Option B: PPA (20-year) | Starting at R 1.9574/kWh |

| Accepted by: | Date: |
| --- | --- |
| Signature: | Position: |